Technical Recommendation 
         Model：BRTIRUS1510A 

 Name： Six-axis robot( 10kg  1500mm）

[image: image1.png]



( No further notice if the specification and appearance are changed due to improvement and other reasons. Thank you for your understanding.)

INTRODUCTION
1.INTRODUCTION OF BRTIRUS1510A 
BRTIRUS1510A six-axis robot is a general-purpose robot. Its shape and structure are compact, and each joint is equipped with a high-precision reducer. The high-speed joint speed can carry out flexible operations in a narrow space, such as handling, stacking, assembly, and other operations. It has a flexible installation method. 
2.APPLICATION ENVIRONMENT
(1) Material handling and stacking

(2) Packaging and assembly

(3) Grinding and polishing

(4) Laser welding

(5) Spot welding

(6) Bending

(7) Cutting / deburring

3.Functions and Features
A. General layout of robot
BRTIRUS1510A adopts a six axis joint robot structure, and six servo motors drive the rotation of six joint axes through reducer and synchronous pulley. It has six degrees of freedom, namely rotation (x), lower arm (y), upper arm (z), wrist rotation (U), wrist swing (V) and wrist rotation (W).

BRTIRUS1510A body joint is made of high-strength aluminum to ensure the high strength, high speed, high precision and high stability of the machine.

BRTIRUS1510A Features：

● the structure of servo motor with RV Reducer and harmonic reducer, which has small volume, large working range, fast speed and high precision, and can be integrated with turntable, slide conveyor chain system and other auxiliary equipment.

●the handheld dialogue operator of the control system is simple and easy to learn, which is very suitable for production.

●the robot body adopts partial internal wiring, which is safe and environmentally friendly.
Motion Control System

The robot motion controller and operating system are BORUNTE control system, with complete functions and simple operation; Standard RS-485 communication interface, USB socket and related software, support extended 8-axis and offline teaching.

C.Reducer

The reducer used on the robot is RV Reducer and harmonic reducer.

The main features of harmonic reducer drive are:                                    

1. The transmission speed ratio is large. The speed ratio range of single-stage harmonic gear transmission is 70~320.

2. High bearing capacity and strong rigidity.

3. High transmission accuracy. The transmission error is only about 1/4 of that of ordinary cylindrical gear transmission. No backlash engagement, small transmission clearance, suitable for reverse rotation.

4. High transmission efficiency and smooth movement. (up to 69% - 96%).

5. Simple structure, few parts and convenient installation.

6. Small size and light weight.

7. It can transmit movement to the confined space.

The main features of RV Reducer transmission are:

1. Compact mechanical structure, light volume, small and efficient;

2. Good heat exchange performance and fast heat dissipation;

3. Simple installation, flexible and light, superior performance, easy maintenance and overhaul;

4. Large transmission speed ratio, large torque and high overload bearing capacity;

5. Stable operation, low noise and durability;

6. Strong applicability, safety and reliability.

D.Servo motor
 The servo motor adopts absolute value motor. Its main features are:

1) Accuracy: realize the closed-loop control of position, speed and torque; The problem of stepping motor out of step is overcome;

2) Speed: good high-speed performance, generally the rated speed can reach 2000~3000 rpm;

3) Adaptability: it has strong overload resistance and can withstand loads three times the rated torque. It is especially suitable for occasions with instantaneous load fluctuations and quick start requirements;

4) Stable: stable operation at low speed, suitable for occasions with high-speed response requirements;

5) Timeliness: the dynamic response time of motor acceleration and deceleration is short, generally within tens of milliseconds;

6) Comfort: fever and noise are significantly reduced. 

E. Working Environment 
1)  power source：380V±10%  50HZ±1%

2)  temperature：0℃-40℃

3)  best temperature：15℃-25℃

4)  relative humidity：20-80%RH ( No condensation )

5)  Mpa：0.5-0.7 Mpa   

                                                                                             4. Main Technical Parameters
	Items
	Content
	Unit value

	Body parameters
	Maximum operating radius
	1500mm

	
	Max loading
	10KG

	
	Repeat positioning accuracy
	[image: image2.wmf]±

0.05mm

	
	IP code
	wristIP54/ bodyIP50

	
	Weight
	About 130KG

	Motion range
	J1
	-165°~+165°

	
	J2
	-95°~+70°

	
	J3
	-85°~+75°

	
	J4
	-180°~+180°

	
	J5
	-115°~+115°

	
	J6
	-360°~+360°

	Max speed 
	J1
	163°/s

	
	J2
	148°/s

	
	J3
	223°/s

	
	J4
	250°/s

	
	J5
	270°/s

	
	J6
	337°/s


5.Vulnerable Parts 
	
	Name 
	Specifications 
	Brand 

	1
	Timing belt
	Tooth shape: 3M,145 tooth， wide: 6mm
	BORUNTE

	2
	Timing belt
	Tooth shape:3M,100 tooth， wide: 6mm
	BORUNTE


6.Robot Random Data
	
	Name 
	Number 
	Remarks

	1
	Robot operation manual
	1
	BORUNTE SYSTEM


Equipment Maintenance 
See the robot maintenance manual for equipment maintenance.

